# Stove Use Monitor Arduino sprint

SUM requirements:

The following requirements were set

- Battery operated monitoring

- Portable (System fitted to a cookstove and movable)

- Measure temperature of cookstove, to measure when in use, fuelling patterns, temperature/cooking profiles.

- Data recording: Either wireless or through sd card; ideally both

- Maintenance: Battery change every 2 weeks

Feature List for sprint:

- Battery operated monitoring (Done)

- Sense temperature from a thermocouple (Done)

- Transfer data over GSM to a remote server (Done)

- Power measurements (2 weeks operation) (Done)

Features outside of scope for sprint:

- Portability/packaging

- Writing/buffering to SD card

- Web interface/Data representation/displaying

- Deployment on cookstove

Overview:

- A battery powered Arduino MKR1400 takes a temperature reading from a K-type thermocouple (via a MAX31850 breakout board). See figure 1.

- If the temperature is different from a previous temperature reading by more than 0.5C, the temperature is transmitted using the onboard GSM chip to a remote MQTT broker.

- A remote server subscribes to the MQTT broker and saves the data sent by the MKR1400 to a CSV file (node-red is used for the server-side application).

Figure 1 Prototype system

Figure 2 Data streaming in from Node-Red

Low power optimisation:

- SIP: The GSM modem is the largest energy consumer so SIP is used to reduce the number of transmissions

- Sensor switching: To reduce power for sensing, the sensor is only switched on long enough to take a reading.

- GSM switching: The GSM module is only left on long enough to make a transmission

- Sleep: When not doing anything, the node is put into a low power sleep state, the node wakes every minute with an interrupt from the RTC.

Figure 3 Thermocouple power on/off

Power consumption

- The node is powered by a 3.7V li-lo battery; a range of capacities are available (up to around 10,000mAh).

- Power consumption (timing):

  - Idle: 14mA (35S during a send period; 59.8S otherwise)

  - Sensing and processing: 24.5mA (134ms)

  - Transmitting: 80-150mA (25 seconds)

- Limitations:

  - We need to make more detailed measurements, especially during the transmitting period where the current draw changes frequently---code needs to be written in LabVIEW to read from the multimeter (est. 1-week job)

  - Lower power sleep states should be possible see the discussion regarding the mkr1200: https://forum.arduino.cc/index.php?topic=499101.0. The k1200 can achieve <500uA idle. I’m not sure why this is not the case for the MKR1400. I’ve posted on the MKR1400forum to ask for advice / tricks. I’ve also asked Arduino support.

Lifetime estimate

- Assumptions

  - Only 5% packets are transmitted

  - 10,000mAh battery

- We should achieve around 16 days with this setup.

- If power was reduced to a similar region as the MKR1200, then we would achieve approx. 72days+ lifetime

Hardware fritzing diagram:

Figure 4 Fritzing diagram of MAX31850 connected to Arduino MKRGSM 1400

SVN Location: svn+ssh://cogentee.coventry.ac.uk/svn/HELP/SystemDevelopment/Software/Cookstove/SUM/Connections.fzz

Code:

SVN Location: svn+ssh://cogentee.coventry.ac.uk/svn/HELP/SystemDevelopment/Software/Cookstove/SUM

Installation / running guide

- If setting up from scratch install MQTT on a remote server following the guide here: https://diyprojects.io/mqtt-mosquitto-communicating-connected-objects-iot/#.WvGWMi-ZPs-

- Install node-red to a laptop/server: https://nodered.org/docs/getting-started/installation and https://www.digitalocean.com/community/tutorials/how-to-connect-your-internet-of-things-with-node-red-on-ubuntu-16-04 (step 3-5)

- Checkout the code from SVN

- In the svn folder copy node-red flow to flow directory (~/.node-red/lib/flows) then import and deploy (be sure to modify file location in node-red flow)

- Connect up an Arduino following the hardware fritzing diagram (required components: MKR1400, MAX41850, k-type thermocouple, 4.7k resistor)

- Open SUM.ino in the Arduino IDE

- Modify SUM.h to set MQTT server address (line 22), SIP threshold (line 30), turn debug on/off (line 2). NOTE: Debug must be off when running from battery

- Upload code to Arduino, power and run from a LiLo battery

- Check data is being saved

Long-term trial run

- The node will be deployed in a conservatory for two-weeks from the 10th May 2018.

- The long-term test will test 1) the functionality of the node, 2) the battery life of the node.

- Live data will be viewable from: http://159.65.25.153:1880/#flow/b2c9546.5c5f728

- Note: With SIP being used, you may need to wait an hour or more to see data come in. (See Figure 2 as an example of what you should see).

- The data will be saved in the server’s home directory (“/home/ross/RossTest.csv”)

Things to do

CJC epoxy coating in enclosure

Packaging design

  - Battery change

  - Portability

  - Attachment to cookstove

  - Fixture of thermocouple

  - Fire resistance

  - IP rating/suitability for environment

Battery size with heat

Battery voltage measurement

Lower power MKR1400

Test SIP with a cooking profile

Adaptive sample rate

Longevity test---simulated day---number of cooking events

Local data buffering in flash memory to reduce transmissions

Calibration

Power measurement

Field test

Purchasing for five systems

Purchasing Ibuttons

SUM: Robustness, data reliability, and longevity